Richard Gadberry

Introduction to GIS Programming

May 9, 2018

Final Project

Summary

In California, raw milk production tends to be concentrated in the Central San Joaquin Valley in counties such as Kings, Kern, Merced, Madera, Fresno, Stanislaus, and Tulare, while a majority of the state’s population, and consequently its dairy consumers, reside in much denser urban areas clustered in the southern portion of the state as well as to the north-east.  Typically, producers transport raw milk to processing plants by truck, where the milk is pasteurized and either packaged as drinking milk or processed into other products such as cheese and butter.   Products intended for interstate or international shipment often undergo transshipment from truck to rail or truck to ship at intermodal transportation points.

The script analysis seeks to identify individual milk processing plants which fulfill a set of geographic and attribute criteria.  Specifically, the analysis seeks to identify plants processing fluid milk within core dairy producing regions.  Additional criteria include proximity to established trucking routes.  The script identifies the nearest intermodal transportation point to each selected plant and calculates the distance between the points.

Source Data

The project utilizes data and shapefiles from sources including the California Department of Food & Agriculture and California Department of Transportation.

The California Department of Food & Agriculture publishes registered Dairy Processing Plants on the Department website. The information includes business names and addresses as well as products manufactured.   California Department of Food & Agriculture also publishes a yearly Agricultural Statistic Review.  The 2017 Annual Review includes a report tabulating the number of dairy cows per county.  These statistics served as the basis for identifying “dairy regions” in the script analysis.

A polygon layer of California Counties utilized in the analysis was obtained from California Department of California Department of Forestry and Fire Protection (Cal Fire).

California Department of Transportation provides a linear feature layer on the Department website delineating approved trucking routes.

California Department of Transportation also publishes a point feature layer online, designating intermodal freight shipping points where freight can be transferred between trucks, rail, and or ships.

Methods

The script initially draws the county feature class as a layer.  A query selecting by name selects the identified dairy producing counties.  These include Kern, Tulare, Kings, Fresno, Madera, Merced, Stanislaus, and San Joaquin counties.  A chief limitation of this component stems from the low resolution inherent in this methodology.  Categorizing entire counties as dairy regions may overlook highly irregular distributions of dairy production within those counties.  Identifying individual dairy farms and clusters of dairy farms offers the promise of greater resolution.  Unfortunately for research purposes, California Department of Food & Agriculture does not publicly disclose the location of specific registered farms for bio-security reasons.

The script also draws the milk processing plants feature class as a feature layer, with a query selecting only those plants producing fluid milk.  Utilizing Select by Location, the script identifies those milk producing plants within the selected counties.  Next, the Copy Features tool copies the selected plants to a new feature class.

The truck routes feature class is then drawn as a feature layer.  Utilizing the Buffer Tool, a buffer is generated around the truck routes layer.  A buffer distance variable was assigned to a distance of 5,000 feet.   Next, the Select by Location tool identifies the previously selected milk producing plants that fall within the buffer layer.   The resultant output produces a feature class of plants producing fluid milk, located within selected dairy producing counties, and located within 5,000 feet of an approved trucking route.

Employing the Near Tool, the script generates attribute values within the selected milk plants, including the object_id (Nearid in the attribute table) of and distance to the nearest Intermodal Transportation Point.  Once this data is generated, the script joins the selected milk plants to the Intermodal Transportation Points Layer utilizing the shared identification fields (Nearid and Objectid).  This generates an attribute table within the milk plants layer that includes the name of each plant, plant address, the name of and distance to the nearest intermodal facility and the county of the corresponding intermodal facility.

The script employs an Insert Cursor to insert the values from the table join to the newly created table.  Within the Insert Cursor, a Search Cursor, iterates through the desired fields in the joined table.

Implementing the Search Cursor involved a fair amount of troubleshooting.   The cursor continued to return an error of “queue empty”.  After double checking field names and inserting print statements to print the exact names of fields to check for errors in transcription, ruled spelling errors as the source of the bug.  Next, I added a query to eliminate records with null values, but the error persisted.  Eventually, I noticed that the field_list utilized in the Search Cursor referenced the field names of the Select_Buff_Layer as opposed to the Select_Buffer_Milk feature class.  Once the field names in the list were changed to reference the feature layer, the Search Cursor executed properly.

Results

Ultimately, the script identified four milk producing plants matching the selected criteria.  Two of the plants operate in Fresno, one in operates in Modesto, and the final plant operates in Hanford.  Three of the plants share a nearest intermodal transit point in Fresno and the fourth stands nearest the Lathrop intermodal facility.  The script successfully copied these field values to a new stand alone table.