Sacramento Railyards Project Documentation

Summary:

The primary data frame and subsequent data collections, for this initial phase of the project, were all initially manually adjusted to the WGS_1984_Web_Mercator_Auxiliary_Sphere projected coordinate system for publishing toward ArcGIS Portal. The main objective for this phase of the project was to automate the raster processing effort by using python scripts to expedite the data processing into MS SQL server and ultimately into ArcMap.

Geo PDF files provided the historical component for this project, that is, utilizing import data prior to recent GIS technological developments. Geo PDF’s are becoming a prevalent means of distributing georeferenced spatial data (either vector or raster based or a combination of both) because of the portability characteristic. This is a practicality within the USGS as further distributed data is rendered into this format as it adds a coordinate transformation matrix and other metadata.

Additional spatial data was manually incorporate into the project for inclusion into MS SQL server and ArcMap to enhance potential analysis. All of the manually imported data can be automated within a future project phase via python scripts, by programmatically traversing a particular folder, and any associated subdirectories – as demonstrated within this initial project objective.

Purpose:

- Provide data for analysis by loading data into ArcGIS Desktop or Pro with python automation scripts

- Collect and process historical data to share a story via maps or ArcGIS portal

Description:

Python Scripts:

The flow diagram below shows the general steps that produced the two python scripts that were used in this project for processing image files. The actual code is located at the end of this document for reference.

The database transfer for this initial phase was accomplished manually by using ArcCatalog to load data into MS SQL server.

The first script was created to process multiple geoPDFs from a common folder. These files represent historical data that is distributed by the USGS within this format to facilitate portability. The main engine of work for this script is having the capacity to traverse the contents of a folder and potential subdirectories by using the python built-in function in the OS module that is provided by OS.walk(). This method generates the file names of a directory which produces a 3-tuple in the form of: (dirpath, dirnames, filenames).

The main sections for this script are partitioned as follows:

- Traversing input folder

- Filter for geoPDF files

- Remove prior TIFF files from output folder

- Convert PDF to TIFF

- Provides pyramids to improve the display performance

The script execution via the IDLE shell output is shown below:

Python 2.7.14 (v2.7.14:84471935ed, Sep 16 2017, 20:25:58) [MSC v.1500 64 bit (AMD64)] on win32

Type "copyright", "credits" or "license()" for more information.

>>>

================== RESTART: Q:\prj_New_Neighbors\source - py\pdf_2_tiff.py ==================

Starting PDF to TIFF Conversion.

Exported Q:\prj_New_Neighbors\data\imagery\TIFFs\us10e CA_Sacramento_East_20150324_TM_geo.tif

Exported Q:\prj_New_Neighbors\data\imagery\TIFFs\us10w CA_Sacramento_West_20150324_TM_geo.tif

Exported Q:\prj_New_Neighbors\data\imagery\TIFFs\us9a CA_Sacramento East_294878_1954_24000_geo.tif

Exported Q:\prj_New_Neighbors\data\imagery\TIFFs\us9b CA_Sacramento East_294882_1967_24000_geo.tif

Exported Q:\prj_New_Neighbors\data\imagery\TIFFs\us9c CA_Sacramento East_101948_1992_24000_geo.tif

Exported Q:\prj_New_Neighbors\data\imagery\TIFFs\us9d CA_Sacramento East_294880_1967_24000_geo.tif

Exported Q:\prj_New_Neighbors\data\imagery\TIFFs\us9e CA_Brighton_295949_1911_31680_geo.tif

Exported Q:\prj_New_Neighbors\data\imagery\TIFFs\us9f CA_Sacramento East_294879_1954_24000_geo.tif

Exported Q:\prj_New_Neighbors\data\imagery\TIFFs\us9g CA_Sacramento East_294881_1967_24000_geo.tif

Exported Q:\prj_New_Neighbors\data\imagery\TIFFs\us9h CA_Brighton_295950_1911_31680_geo.tif

Exported Q:\prj_New_Neighbors\data\imagery\TIFFs\us9i CA_Sacramento East_294877_1949_24000_geo.tif

Exported Q:\prj_New_Neighbors\data\imagery\TIFFs\us9j CA_Sacramento West_294888_1967_24000_geo.tif

Sucessful batch run - PDF conversion complete.

>>>

The purpose of the second python script is to process raster images and yield a clipped composition image for the extent of the study area. The objective of providing a clipped raster is to provide portability by eliminating data that is external to the extent of the study area. One raster is processed by this script to save on processing time for the development cycle. However, multiple raster files can also be processed by utilizing the OS.walk method as demonstrated in the first script.

Most of the coding in this script is for the development cycle feedback, which renders the composite and clipped images into a pre-existing ArcGIS map document for validation. However, a production version can omit this section of code.

The main sections for this python script are partitioned as follows:

- Remove pre-existing files from output folder

- Select valid raster bands to process

- Remove pre-existing layers from ArcGIS map document

- Composite band processing

- Clip composite band processing into the extent of study area

- Provide image feedback via ArcGIS map document

The script execution via the shell output is shown below:

=============== RESTART: Q:\prj_New_Neighbors\source - py\raster_composite.py ===============

Starting Raster Composition Module...

Removing previous folder files:

landsat_jan_2014_compositebands.tfw

landsat_jan_2014_compositebands.tif

landsat_jan_2014_compositebands.tif.aux.xml

landsat_jan_2014_compositebands.tif.ovr

landsat_jan_2014_compositebands.tif.xml

landsat_jan_2014_compositebands_clip.rlt

landsat_jan_2014_compositebands_clip.tfw

landsat_jan_2014_compositebands_clip.tif

landsat_jan_2014_compositebands_clip.tif.aux.xml

landsat_jan_2014_compositebands_clip.tif.ovr

landsat_jan_2014_compositebands_clip.tif.xml

Valid input files:

LC08_L1TP_044033_20140113_20170307_01_T1_sr_band1.tif

LC08_L1TP_044033_20140113_20170307_01_T1_sr_band2.tif

LC08_L1TP_044033_20140113_20170307_01_T1_sr_band3.tif

LC08_L1TP_044033_20140113_20170307_01_T1_sr_band4.tif

LC08_L1TP_044033_20140113_20170307_01_T1_sr_band5.tif

LC08_L1TP_044033_20140113_20170307_01_T1_sr_band6.tif

LC08_L1TP_044033_20140113_20170307_01_T1_sr_band7.tif

Creating raster composition:

Removing previous layer ==> clipRaster

Removing previous layer ==> fullRaster

Adding full raster into map document

Adding clip raster into map document

Completed raster composition process.

>>>

Challenges:

- Acquiring data - not easy to obtain free data

- Level of effort - within time constraints

- Learning curve using the arcpy python library

The resolution to the learning curve for the arcpy library was read the online documentation for both python and ArcGIS for syntax and method usage.

Data Sources:

Landsat	:

https://landsatlook.usgs.gov/

City of Sacramento – Open Portal:

http://data.cityofsacramento.org/

National Railways:

https://catalog.data.gov/dataset/2015-rails-national-geodatabase

Earth Resources Observation and Science :

https://eros.usgs.gov/

USGS:

http://historicalmaps.arcgis.com/usgs/

References:

Python website:

www.python.org

Esri:

www.esri.com

ArcGIS:

https://desktop.arcgis.com/en/arcmap/10.5/analyze/arcpy-classes/alphabetical-list-of-arcpy-classes.htm

Stack Overflow:

stackoverflow.com

Sacramento Railyards Specific Plans:

http://railyards.com/Portals/0/Documents/ProposedRailyardsSpecificPlanDRAFT.pdf?ver=2016-08-18-213326-060

Jennings, Nathan. A Python Primer for ArcGIS Workbook II, 2015

Lutz, Mark. Learning Python, 5th Ed. Beijing: O’Reilly Media, Inc., 2013